HDMI to VGA Converter Lab Manual

Takatsu Naoaki 015746144

CECS 461 Spring 2019

14, May 2019

- Install Vivado 2016.4 Edition and SDK from Xilinx

- Follow the steps in the link to download and install “xmdterm.tcl” from Xilinx

- Follow the steps in the link to download vivado-library to use custom IPs from Github

- Download HDMI input demo project zip file from Xilinx and follow all the steps provided

- Follow steps 1 to 2 for Vivado (not SDK Hardware Handoff) shown on this link to create and open demo project

- Go to Project Manager > Project Settings > IP category > Repository Manager. Now click the green plus mark and navigate to the location where you have saved your “vivado-library” folder

- Go to Tools > Report > Report IP Status

- Click the Upgrade Selected button on the IP Status tab

- *Optional: you may remove GPIO_leds block and its external pin since it’s not used for this project. After removing from block diagram, remove all led related codes from hdmi_in_wrapper.vhd

- Go to Synthesis > Run Synthesis to run synthesis

- Go to Implementation > Run Implementation to run implementation

- Go to Program and Debug > Generate Bitstream to generate bitstream

- Go to File > Launch SDK

- Remove all unrelated files in Project Explorer tab if necessary

- Go to File > Import > General > Existing Projects into Workspace and navigate to the sdk folder.

- Import the HDMI_IN folder

- Go to File > New > Application Project

- Type up a name for the application and click Next

- Choose Empty Application and click Finish

- Open HDMI_IN > src and copy all folders and C files (excluding files such as lscript.ld, README.txt, and Xilinx.spec)

- Open your newly created application folder > src and paste the files

- Replace the contents for video_demo.c with the provided one or simply close SDK and replace the file itself

- Click the save all button and check for any error messages. If any error messages show up, try redoing all the steps explained above

- Connect your Zybo board via micro USB to USB cable and power it on. Makes sure that the 2 header pins are set to USB power and JTAG

- Power on the board and go to Xilinx > Program FPGA

- Click Program to program FPGA

- Navigate to SDK Terminal and press the green plus button

- Select the detected port and enter the following information for the settings

- Click OK to connect the Zybo board to the terminal

- Right click your application project and go to Run As > Launch on Hardware GDB

- Connect HDMI and VGA cables to the Zybo board and the other ends to the HDMI source and display device respectively

- The display device should show 640 x 480 resolution from the HDMI source by default

- Review the options shown on the serial terminal and change the dip switch value to adjust the display resolution settings

- Once the dip switches are set to the proper value, press button 2 on the Zybo board to change the display resolution

- Test out all the other menus shown on the terminal to see if it works properly

- Once finished, power off the Zybo Board and unplug the cable from the computer

- Shut down both SDK and Vivado 2016.4